| Source Device | Source Port | Destination Device | Destination Port | Purpose |
| --- | --- | --- | --- | --- |
| EDGE-RTR | G0/1 | MLS-1 | G0/1 | Internal network connection |
| EDGE-RTR | S0/0/0 | BRANCH-RTR | S0/0/0 | WAN link |
| MLS-1 | Fa0/1 | CORE-1 | G0/1 | Core uplink |
| MLS-1 | Fa0/2 | CORE-2 | G0/1 | Core uplink |
| CORE-1 | Fa0/23 | CORE-2 | Fa0/23 | EtherChannel Link 1 |
| CORE-1 | Fa0/24 | CORE-2 | Fa0/24 | EtherChannel Link 2 |
| CORE-1 | Fa0/1 | HR-SW | G0/1 | HR uplink |
| CORE-1 | Fa0/2 | IT-SW | G0/1 | IT uplink |
| CORE-1 | Fa0/3 | MGMT-SW | G0/1 | Management uplink |
| CORE-2 | Fa0/1 | GUEST-SW | G0/1 | Guest uplink |
| CORE-2 | Fa0/2 | FIN-SW | G0/1 | Finance uplink |
| CORE-2 | Fa0/3 | SALES-SW | G0/1 | Sales uplink |
| CORE-2 | Fa0/4 | SERVER-SW | G0/1 | Server Farm uplink |
| SERVER-SW | Fa0/1 | DNS-SRV | Fa0 | DNS Service |
| SERVER-SW | Fa0/2 | DHCP-SRV | Fa0 | DHCP Service |
| SERVER-SW | Fa0/3 | WEB-SRV | Fa0 | Web Service |
| SERVER-SW | Fa0/4 | FILE-SRV | Fa0 | File Storage Service |
| BRANCH-RTR | G0/0 | BRANCH-SW | G0/1 | Branch LAN |
| BRANCH-SW | Fa0/1 | BRANCH-PC-1 | Fa0 | Branch workstation |
| BRANCH-SW | Fa0/2 | BRANCH-PC-2 | Fa0 | Branch workstation |
| BRANCH-SW | Fa0/3 | BRANCH-PC-3 | Fa0 | Branch workstation |
| BRANCH-SW | Fa0/4 | BRANCH-PRINTER | Fa0 | Shared printer |

Note: All switch-to-switch links will operate as trunk links. CORE-1 and CORE-2 are connected using an LACP EtherChannel. Unused ports will be assigned to VLAN 999 and administratively shut down.